Lab: Sensor Fusion and Bayesian State Estimation

Objective

Implement and compare sensor fusion approaches for a mobile robot, framing multi-sensor integration as variational inference under a generative model. You will design observation models for multiple sensor modalities, implement precision-weighted fusion, and analyze how Active Inference naturally handles sensor degradation.

Prerequisites

Understanding of Bayesian state estimation (prior, likelihood, posterior)  Familiarity with common robotic sensors (LIDAR, cameras, IMUs, encoders)  Basic linear algebra and probability theory   Part 1: Sensor Observation Models

For a mobile robot navigating an indoor environment, specify the observation model for each sensor:

Wheel encoders : Model the relationship between encoder ticks and robot displacement. Include systematic errors (wheel slip, diameter mismatch) and random noise.  2D LIDAR : Model range measurements as a function of robot pose and map geometry. Include beam noise, max-range returns, and short-range reflections.  IMU : Model accelerometer and gyroscope readings. Distinguish between bias drift (slowly varying) and white noise components.  RGB camera : Describe qualitatively how visual features (corners, edges) provide position information. What is the observation model structure?   For each sensor, specify: measurement rate, noise characteristics, and failure modes.

Write your response here  Part 2: Precision Weighting and Sensor Reliability

In Active Inference, each sensor channel has an associated precision (inverse variance) that determines its influence on belief updates.

Design a scenario where the robot traverses three zones:  Zone A : Well-lit, textured hallway (camera reliable, LIDAR reliable)  Zone B : Glass-walled corridor (LIDAR unreliable due to specular reflections, camera reliable)   Zone C : Dark warehouse section (camera unreliable, LIDAR reliable)

For each zone, specify qualitative precision values for each sensor and explain how the Active Inference agent automatically adjusts its reliance on different modalities.

How does this compare to a fixed-gain Kalman filter that cannot adjust sensor weights online?

Write your response here  Part 3: Prediction Error Analysis

Trace a perception cycle for the robot entering Zone B (glass corridor):

Prior belief : Robot at position (5.0, 2.0) with heading 0 rad, low uncertainty.  Predicted observations : Expected LIDAR ranges based on wall map; expected camera features based on hallway model.  Actual observations : LIDAR returns maximum range (beams pass through glass); camera sees corridor features normally.  Prediction errors : Compute qualitative errors for each sensor channel.  Belief update : How does the agent resolve the conflict? What role does precision play?   Write your response here  Part 4: Active Perception

Design a scenario where the robot actively moves to reduce perceptual uncertainty:

The robot detects an ambiguous object at the edge of its LIDAR range. Two hypotheses: (a) it is a person, (b) it is a pillar.  What actions could the robot take to disambiguate? (e.g., move closer, change viewpoint, activate a different sensor)  Frame this as expected free energy minimization: which action is predicted to yield the greatest reduction in uncertainty (epistemic value)?  Write pseudocode for an active perception routine:   function  active_perception ( belief, hypotheses, available_actions ):   for  each  action  in  available_actions :   #  Compute  expected  observation  under  each  hypothesis   #  Compute  expected  information  gain   select  action  with  highest  expected  information  gain   return  selected_action   Write your response here  Part 5: Sensor Fusion Algorithm Design

Write pseudocode for a complete perception update step that fuses all four sensor modalities:

function  perception_update ( prior_belief, observations, precisions, model ):   #  1.  Generate  predictions  from  prior  belief   #  2.  Compute  prediction  errors  for  each  sensor   #  3.  Weight  errors  by  sensor  precisions   #  4.  Update  belief  ( gradient  descent  on  free  energy )   #  5.  Update  precision  estimates  ( optional :  learn  sensor  reliability )   return  posterior_belief   Write your response here  Summary Table

Sensor  Observation Model  Typical Precision  Failure Mode  AI Adaptation      Wheel encoders  Odometry integration    Wheel slip      2D LIDAR  Range-bearing model    Specular reflection      IMU  Inertial kinematics    Bias drift      RGB Camera  Feature projection    Low light       References